# PHYSICS LABORATORY 101

Measurement, Data Treatment, Uncertainty Analysis and Scientific Reporting

Student Reference Manual for First-Year Undergraduate Physics

Version 2 - Corrected Edition

# Table of Contents

| Section | Topic |
| --- | --- |
| - | Purpose of this manual |
| 1 | The meaning of a measurement |
| 2 | Accuracy, precision, random effects and systematic effects |
| 3 | Raw data, data integrity and the laboratory notebook |
| 4 | Treating repeated measurements |
| 5 | Absolute, relative and percentage uncertainty |
| 6 | Measurement error versus uncertainty |
| 7 | Significant figures and rounding |
| 8 | Propagation of uncertainty |
| 9 | Complete uncertainty example: density of a cylinder |
| 10 | Correct data treatment and data manipulation |
| 11 | Graphical data analysis |
| 12 | Data transformation and linearisation |
| 13 | The physical meaning of slope, intercept and residuals |
| 14 | How to compare experiment with theory or a reference value |
| 15 | How to write a proper error analysis |
| 16 | Correct reporting of numerical results |
| 17 | Correct reporting of tables and graphs |
| 18 | Complete reporting example: simple pendulum |
| 19 | Recommended laboratory report structure |
| 20 | Common beginner mistakes |
| 21 | Quick-reference formula sheet |
| 22 | Final analysis and reporting checklist |
| 23 | Essential experimental mindset |
| - | Recommended introductory reference |

# Purpose of this manual

Experimental physics is not primarily about obtaining a number that agrees with a textbook. It is about answering a physical question using measurements and deciding what the evidence justifies.

A complete experiment normally follows the sequence

Physical question → measurement → raw data → data treatment → uncertainty analysis → model → interpretation → conclusion.

This manual develops that sequence step by step. The emphasis is on practical judgement, correct handling of data, meaningful uncertainty analysis, and clear scientific reporting.

## Learning outcomes

After using this manual, you should be able to:

- identify the physical quantities and units involved in an experiment;

- recognise instrument resolution and important measurement limitations;

- distinguish accuracy from precision;

- distinguish random variation from systematic effects;

- record raw data correctly and preserve traceability;

- calculate a mean, sample standard deviation, and standard uncertainty of the mean;

- distinguish uncertainty from measurement error;

- calculate absolute, relative, and percentage uncertainties;

- propagate uncertainty through sums, differences, products, quotients, and powers;

- identify which measured quantity dominates the uncertainty in a calculated result;

- distinguish raw data from processed data;

- transform data correctly while propagating the corresponding uncertainty;

- construct and interpret scientific graphs and error bars;

- obtain physical parameters from fitted slopes and intercepts;

- compare an experimental result with a reference value in a scientifically meaningful way;

- write a specific and quantitative error analysis; and

- report final results with justified significant figures and units.

# 1. The meaning of a measurement

Suppose the length of an object is written as

L=25.3 cm.

This gives a numerical value and a unit, but it does not tell the reader how well the value is known. A more informative experimental statement might be

L=25.30±0.05 cm.

The value 25.30 cm is the measured estimate. The value 0.05 cm represents an uncertainty associated with that estimate.

Key idea. A measured value without information about its uncertainty is often incomplete.

## 1.1 Units are part of the result

A statement such as L=25 is incomplete if L represents a length. The unit must be stated, for example

L=25 cm.

Units are also a powerful check on calculations. If

v=dt,

then

[v]=m s−1.

For example, a value of v reported in m s−2 would signal an error in the calculation or formula because velocity must have dimensions of length divided by time.

## 1.2 Instrument resolution

Every instrument has finite resolution. Typical examples are shown below; the actual value must always be read from the instrument used in the laboratory.

| Instrument | Typical resolution |
| --- | --- |
| Metre ruler | 1 mm |
| Vernier caliper | 0.1 mm or better |
| Micrometer screw gauge | 0.01 mm |
| Digital balance | 0.01 g |
| Digital stopwatch | 0.01 s display resolution |

Instrument resolution is not automatically equal to the total measurement uncertainty. A stopwatch may display 0.01 s, but a manually operated stopwatch can have a much larger uncertainty because of reaction-time variation.

Before using an instrument, ask:

- What quantity is measured?

- What is the unit?

- What is the resolution?

- Is zero adjustment required?

- Is calibration relevant?

- What practical effects may limit the measurement more strongly than the instrument resolution?

# 2. Accuracy, precision, random effects and systematic effects

## 2.1 Precision

Precision describes how closely repeated measurements agree with one another. Consider

10.21, 10.22, 10.21, 10.23 cm.

The scatter is small, so the measurements are precise.

## 2.2 Accuracy

Accuracy describes closeness of a measured result to an appropriate reference quantity value. If the relevant reference value were

10.50 cm,

then the measurements above would be precise but not accurate.

Key idea. High precision does not guarantee high accuracy.

| Precision | Accuracy | Interpretation |
| --- | --- | --- |
| High | High | Desirable measurement |
| High | Low | Possible systematic effect |
| Low | Mean near reference | Large random scatter; individual readings are imprecise |
| Low | Low | Poor-quality measurement |

## 2.3 Random effects

Random effects produce unpredictable variation from one trial to another. Examples include:

- reaction-time variation;

- electrical noise;

- vibration;

- small changes in alignment or positioning;

- environmental fluctuations.

Random effects normally appear as scatter in repeated measurements.

## 2.4 Systematic effects

Systematic effects tend to shift measurements consistently. Examples include:

- an incorrectly zeroed balance;

- calibration offset;

- consistent parallax;

- an incorrect scale;

- unaccounted heat loss that systematically biases an inferred thermal quantity;

- unaccounted friction that systematically biases a result obtained from an idealised mechanical model.

Suppose a balance reads 0.50 g too high. Repeated measurements may be extremely consistent, but taking more measurements will not remove that offset.

Key idea. Repetition helps characterise random scatter; it does not automatically correct systematic bias.

# 3. Raw data, data integrity and the laboratory notebook

## 3.1 Raw data versus processed data

Suppose the time for ten pendulum oscillations is measured five times.

| Trial | t10 / s |
| --- | --- |
| 1 | 14.18 |
| 2 | 14.22 |
| 3 | 14.16 |
| 4 | 14.20 |
| 5 | 14.19 |

These are raw data. The mean calculated from them is processed data.

Do not replace the raw observations by only the processed value. The original measurements contain information about scatter and experimental quality.

## 3.2 Preserve traceability

A laboratory notebook should contain enough information for the experiment to be understood later. Record:

- date and experiment title;

- objective;

- apparatus and instrument resolution;

- experimental arrangement;

- raw measurements and units;

- unusual observations;

- changes to the procedure;

- preliminary calculations.

If a value was recorded incorrectly, cross it out once and write the correction. Do not erase the original entry so completely that the history of the record is lost.

## 3.3 Correct handling of a suspected outlier

Suppose repeated times are

2.14, 2.09, 2.12, 2.11, 2.47 s.

The value 2.47 s appears unusual. It must not be deleted merely because it is far from the others.

Investigate whether there is a documented reason for exclusion. For example:

Trial 5 was excluded because the cart visibly struck the side of the track, so the intended experimental condition was not satisfied.

This is defensible. The statement “Trial 5 was removed because it was too far from the other values” is not sufficient.

# 4. Treating repeated measurements

## 4.1 Mean

For N repeated measurements x1,x2,…,xN, the arithmetic mean is

x=1Ni=1Nxi.

### Worked example: cylinder diameter

Measured values are

12.42, 12.39, 12.45, 12.43, 12.41 mm.

The mean is

D=12.42+12.39+12.45+12.43+12.415=12.420 mm.

Do not round aggressively at this stage. Final rounding is performed after the uncertainty is known.

## 4.2 Sample standard deviation

The sample standard deviation is

s=i=1Nxi−x2N−1.

For the diameter example:

| Di / mm | Deviation / mm | Squared deviation / mm² |
| --- | --- | --- |
| 12.42 | 0.000 | 0.0000 |
| 12.39 | -0.030 | 0.0009 |
| 12.45 | +0.030 | 0.0009 |
| 12.43 | +0.010 | 0.0001 |
| 12.41 | -0.010 | 0.0001 |

Hence

s=0.00204≈0.022 mm.

The standard deviation describes the typical scatter of individual measurements around their mean.

## 4.3 Standard uncertainty of the mean

For independent repeated measurements under comparable conditions,

ux=sN.

For the example,

uD=0.0225≈0.010 mm.

Thus, considering the statistical contribution alone,

D=12.420±0.010 mm.

Other uncertainty sources, such as instrument resolution or calibration, may still need to be considered.

### Do not confuse s and ux

- s answers: How widely do individual measurements scatter?

- ux answers: How precisely has the mean been estimated from the repeated measurements?

# 5. Absolute, relative and percentage uncertainty

For

L=50.0±0.5 cm,

the absolute uncertainty is

uL=0.5 cm.

The relative uncertainty is

uLL=0.550.0=0.010.

The percentage uncertainty is

100uLL=1.0%.

Relative uncertainty is especially useful when comparing the precision of measurements having different magnitudes or units.

# 6. Measurement error versus uncertainty

The terms error and uncertainty should not be used as if they mean exactly the same thing. An error is a signed discrepancy relative to a specified reference quantity value; an uncertainty is a non-negative quantity that characterises how widely values that could reasonably be attributed to the measurand may be dispersed.

When an appropriate reference quantity value is available, measurement error can be expressed conceptually as

measurement error=measured value−reference value.

Uncertainty describes how well the measured value is known.

For example, in

g=9.75±0.19 m s−2,

the quantity

0.19 m s−2

is an uncertainty. It should not automatically be called “the error”.

The traditional phrase error analysis remains common in undergraduate laboratories. In this manual it refers broadly to the analysis of uncertainties, discrepancies, random variation, systematic effects, and experimental limitations.

## 6.1 Percentage difference from a reference value

If

gmeas=9.65 m s−2

and

gref=9.81 m s−2,

then

% difference=100gmeas−grefgref

and therefore

% difference≈1.63%.

This quantity is often called percentage error in introductory work. It is not the same as percentage uncertainty.

# 7. Significant figures and rounding

A calculator can display many digits, but an experiment rarely justifies reporting all of them.

Suppose

x=12.43782941,

with uncertainty

ux=0.16.

Reporting

12.43782941±0.16

is misleading. A suitable result is

x=12.44±0.16.

A practical procedure is:

- retain extra digits during intermediate calculations;

- determine the uncertainty;

- round the uncertainty appropriately, usually to one or two significant figures;

- round the measured value to the same decimal place as the uncertainty.

### Example

If

x=8.73614,

and

ux=0.21367,

a suitable final result is

x=8.74±0.21.

Key idea. Calculator precision is not experimental precision.

# 8. Propagation of uncertainty

Many physical quantities are calculated from measurements rather than measured directly. Uncertainty in the inputs therefore produces uncertainty in the calculated result.

Examples include

v=dt,  ρ=mV,  R=VI,  K=12mv2.

The process of determining the uncertainty in a calculated quantity from the uncertainties of the input measurements is called propagation of uncertainty.

The rules in the following sections are first-order approximations for relatively small uncertainties and assume that the uncertainty contributions are independent. If important input quantities are correlated, covariance terms must be included.

## 8.1 Addition and subtraction

If

Q=A+B

or

Q=A−B,

then, for independent inputs,

uQ≈uA2+uB2.

### Worked example: difference between two positions

Suppose

L=x2−x1,

where

x1=12.40±0.04 cm,

and

x2=35.20±0.05 cm.

The measured length is

L=35.20−12.40=22.80 cm.

The uncertainty is

uL=0.052+0.042=0.064 cm.

Hence

L=22.80±0.06 cm.

Incorrect treatment:

0.05−0.04=0.01 cm.

Subtracting measured values does not mean subtracting their uncertainties.

## 8.2 Multiplication and division

For

Q=ABC,

independent relative uncertainties combine to first order as

uQQ2≈uAA2+uBB2+uCC2.

### Worked example: velocity

A cart travels

d=2.00±0.01 m

in

t=1.25±0.03 s.

The velocity is

v=dt=2.001.25=1.60 m s−1.

The relative contributions are

udd=0.012.00=0.005=0.5%,

and

utt=0.031.25=0.024=2.4%.

Therefore

uvv=0.0052+0.0242≈0.0245.

Hence

uv=1.600.0245≈0.039 m s−1.

The final result is

v=1.60±0.04 m s−1.

### What does the error analysis tell us?

The time contributes about 2.4% while the distance contributes only 0.5%. Improving the timing method is therefore more important than greatly improving the ruler.

Key idea. Error analysis should identify the dominant limitation and guide experimental improvement.

## 8.3 Powers

For

Q=An,

the relative uncertainty is

uQQ≈nuAA.

### Worked example: area of a disc

Suppose

r=10.00±0.10 mm.

Then

A=πr2=314.16 mm2.

Because A∝r2,

uAA=2urr=20.1010.00=0.020.

Therefore

uA=314.160.020=6.28 mm2.

A suitable result is

A=314±6 mm2.

The radius has a 1% uncertainty, but the area has approximately a 2% uncertainty because the radius is squared.

## 8.4 General product-of-powers rule

For

Q=AmBnCp,

independent uncertainties propagate as

uQQ2≈m2uAA2+n2uBB2+p2uCC2.

### Worked example: kinetic energy

Suppose

m=0.250±0.002 kg,

and

v=1.80±0.05 m s−1.

The kinetic energy is

K=12mv2=0.405 J.

Because K∝mv2,

uKK=umm2+2uvv2.

The mass contribution is

0.0020.250=0.008,

while the velocity contribution is

20.051.80=0.0556.

Hence

uKK≈0.056,

and

uK≈0.4050.056=0.023 J.

A suitable final report is

K=0.41±0.02 J.

The velocity measurement dominates because v is squared in the kinetic-energy expression.

# 9. Complete uncertainty example: density of a cylinder

This example illustrates the complete sequence

measurement→calculation→uncertainty propagation→uncertainty budget→reporting.

The density of a cylinder is

ρ=mV,

where

V=πD2h4.

Therefore

ρ=4mπD2h.

Suppose

m=56.20±0.05 g,

D=20.00±0.05 mm,

and

h=25.00±0.10 mm.

## 9.1 Treat units consistently

Convert to centimetres:

D=2.000±0.005 cm,

h=2.500±0.010 cm.

The value and absolute uncertainty are converted using the same factor.

## 9.2 Calculate the density

ρ=456.20π2.00022.500≈7.1556 g cm−3.

Keep the extra digits temporarily.

## 9.3 Propagate uncertainty

Since

ρ∝mD−2h−1,

we obtain

uρρ=umm2+2uDD2+uhh2.

Individual contributions are

umm=0.0556.20=0.00089=0.089%,

2uDD=20.0052.000=0.0050=0.50%,

and

uhh=0.0102.500=0.0040=0.40%.

Therefore

uρρ=0.000892+0.00502+0.00402≈0.00646.

Thus

uρ=7.15560.00646≈0.0463 g cm−3.

The final result is

ρ=7.16±0.05 g cm−3.

## 9.4 Uncertainty budget

| Source | Weighted relative-uncertainty term |
| --- | --- |
| Mass term, m | 0.089% |
| Diameter term, D2 | 0.50% |
| Height term, h | 0.40% |

The diameter term is the largest contributor. Improving the balance by an order of magnitude would produce little benefit compared with improving the diameter measurement.

This is an example of quantitative error analysis: the calculation identifies which experimental limitation matters most.

# 10. Correct data treatment and data manipulation

In scientific work, data treatment means legitimate mathematical processing of measured data. Examples include:

- unit conversion;

- averaging repeated measurements under equivalent conditions;

- subtraction of a measured background;

- calculation of derived quantities;

- transformation of variables according to a physical model;

- fitting an appropriate model.

Every transformation should be justified, documented, and reproducible.

## 10.1 Raw data and derived data

Suppose a pendulum measurement gives

t10=14.18 s.

This is raw data. The period is

T=t1010=1.418 s,

which is processed data. If the analysis requires

T2,

then

T2=1.4182=2.0107 s2

is further processed data.

The chain

t10→T→T2

must remain traceable to the original observation.

## 10.2 Propagate uncertainty through every transformation

Suppose

t10=14.18±0.10 s.

Since the number of counted oscillations is exact,

T=t1010=1.418±0.010 s.

Now transform to T2. Since the transformed quantity is the square of T,

uT2T2=2uTT.

Therefore

uT2=2.010720.0101.418≈0.028 s2.

The transformed quantity is therefore

T2=2.01±0.03 s2.

Key idea. If a measured variable is transformed, its uncertainty must also be transformed.

## 10.3 Background subtraction

Suppose a detector measures

Vtotal=2.84±0.03 V,

while an independently measured background is

Vbackground=0.35±0.02 V.

The corrected signal is

Vsignal=Vtotal−Vbackground=2.49 V.

Its uncertainty is

uVsignal=0.032+0.022=0.036 V.

Thus

Vsignal=2.49±0.04 V.

Subtracting the background value without propagating its uncertainty is incorrect.

## 10.4 Averaging is not always appropriate

Suppose repeated measurements under the same condition are

2.10, 2.11, 2.09, 2.10 s.

A mean is meaningful.

Now suppose measurements are deliberately made at different temperatures:

| T / °C | R / Ω |
| --- | --- |
| 20 | 102 |
| 30 | 106 |
| 40 | 110 |
| 50 | 114 |

Averaging the four resistances would destroy the relationship being investigated. Before averaging, ask:

Do these measurements represent repeated estimates of the same quantity under equivalent conditions?

## 10.5 Unacceptable manipulation

The following practices are scientifically unacceptable:

- deleting a value simply because it disagrees with theory;

- choosing only points that improve a graph;

- changing an uncertainty to make a reference value fall inside the uncertainty interval;

- rounding values in a preferred direction;

- forcing a fit through the origin without justification;

- replacing raw data with cleaned values while discarding the originals.

# 11. Graphical data analysis

Graphs are analytical tools. They can reveal proportionality, curvature, offsets, outliers, systematic trends, and disagreement with a model.

## 11.1 Independent and dependent variables

The independent variable is normally the quantity deliberately selected or changed and is usually placed on the horizontal axis. The dependent variable is the measured response and is usually placed on the vertical axis.

A scientific graph should normally include:

- quantity and unit on each axis;

- appropriate scales;

- visible data points;

- uncertainty bars where relevant;

- the fitted relationship;

- a useful caption.

## 11.2 Error bars

If

T=1.418±0.010 s,

the uncertainty may be shown by an error bar. The graph caption should state what the bar represents, for example:

Vertical error bars represent the standard uncertainty in each measured value of T.

## 11.3 Do not join data points dot-to-dot

Experimental data usually contain scatter. If a physical model predicts a linear relationship, fit a line of the form

y=mx+c

instead of connecting successive points.

# 12. Data transformation and linearisation

Many physical relationships are not naturally linear. A physically motivated transformation can convert them into a form suitable for linear fitting.

For a simple pendulum,

T=2πLg.

Squaring gives

T2=4π2gL.

A plot of T2 against L should therefore be linear, with

m=4π2g.

Hence

g=4π2m.

## 12.1 Complete pendulum dataset

Suppose the raw measurements are:

| L / m | t10 / s |
| --- | --- |
| 0.200 | 8.96 |
| 0.300 | 10.99 |
| 0.400 | 12.69 |
| 0.500 | 14.18 |
| 0.600 | 15.55 |

Calculate

T=t1010

and then T2:

| L / m | t10 / s | T / s | T2 / s² |
| --- | --- | --- | --- |
| 0.200 | 8.96 | 0.896 | 0.803 |
| 0.300 | 10.99 | 1.099 | 1.208 |
| 0.400 | 12.69 | 1.269 | 1.610 |
| 0.500 | 14.18 | 1.418 | 2.011 |
| 0.600 | 15.55 | 1.555 | 2.418 |

The first two columns preserve the measured quantities. The last two contain derived quantities.

## 12.2 Extracting physics from the fitted slope

Suppose a fit gives

m=4.033±0.006 s2 m−1.

Then

g=4π24.033≈9.79 m s−2.

Since g∝m−1,

ugg=umm.

Thus

ug=9.790.0064.033≈0.015 m s−2.

Considering this fitted-slope contribution alone,

g=9.79±0.02 m s−2.

Other systematic contributions may still need to be considered before interpreting this as the complete uncertainty.

The full analysis chain is

t10→T→T2→fit→m→g→ug.

# 13. The physical meaning of slope, intercept and residuals

## 13.1 Slope

For

y=mx+c,

the most important question is not merely “What is m?” but

What physical quantity does m represent?

Examples:

### Velocity against time

v=v0+at,

so

m=a.

### Voltage against current

V=RI,

so

m=R.

### Pendulum

T2=4π2gL,

so

m=4π2g.

Slope units provide an important consistency check. For V plotted against I,

[m]=VA=Ω,

which is consistent with interpreting the slope as resistance.

## 13.2 Intercept

If theory predicts c=0 but the fitted line gives a non-zero c, do not automatically force the fit through the origin. A non-zero intercept may indicate:

- zero offset;

- calibration bias;

- an omitted physical contribution;

- random scatter;

- inadequacy of the model.

## 13.3 Residuals

For data point i, the residual is

ri=yi−yfit,i.

Residuals scattered randomly around zero, with no systematic structure, are consistent with the chosen model. A curved or otherwise structured pattern can indicate that the model is inadequate.

# 14. How to compare experiment with theory or a reference value

Suppose

gexp=9.75±0.19 m s−2,

while

gref=9.81 m s−2.

The absolute difference is

9.75−9.81=0.06 m s−2.

This difference is smaller than the quoted experimental uncertainty 0.19 m s−2. If the reference uncertainty is negligible compared with the experimental uncertainty, this simple comparison is a useful introductory heuristic, and a reasonable statement is:

The measured value is consistent with the reference value within the stated experimental uncertainty.

### Poor statement

The theoretical value is 9.81 m s−2 and ours is 9.75 m s−2, so experimental error occurred.

### Better statement

The measured value, g=9.75 m s−2, differs from the reference value by 0.06 m s−2. This difference is smaller than the experimental uncertainty, so the two values are consistent at the precision of this experiment.

A percentage difference alone is not a complete error analysis. If the reference value also has a non-negligible uncertainty, compare the discrepancy with the combined uncertainty rather than with the experimental uncertainty alone.

# 15. How to write a proper error analysis

A useful error analysis should answer four questions:

1. How uncertain is the result?2. What causes that uncertainty?3. Which source matters most?4. How could the measurement be improved?

## 15.1 Poor error analysis

There were human errors during the experiment. Errors may also come from the equipment. More accurate equipment should be used next time.

This statement is vague, non-quantitative, and does not show an understanding of the experiment.

## 15.2 Better example: manual timing

The dominant uncertainty is likely associated with manual timing. Although the stopwatch displays 0.01 s, the observer must react visually to the beginning and end of the motion, so reaction-time variation is substantially larger than the display resolution. Repeating the timing reduces the influence of random variation on the mean, but it does not necessarily remove a systematic timing delay. A photogate or automated video-timing system would reduce this limitation.

## 15.3 Better example: cylinder density

The uncertainty in density is dominated by the dimensional measurements rather than the mass measurement. The mass term is approximately 0.089%, while the diameter term is approximately 0.50% and the height term is approximately 0.40%. Because density depends on D−2, uncertainty in the diameter is amplified. Improving the precision of the diameter measurement would therefore be more effective than replacing the balance with a much more precise instrument.

## 15.4 Better example: pendulum

Manual timing contributes random variation to the measured period. Timing several oscillations rather than a single oscillation reduces the relative influence of reaction time. A systematic contribution may arise if the pendulum length is not measured from the pivot to the centre of mass of the bob. The theoretical relation also assumes a sufficiently small oscillation amplitude. Automated timing, careful determination of the effective pendulum length, and small reproducible release angles would improve the measurement.

## 15.5 Avoid the phrase “human error” unless the mechanism is identified

Poor:

Human error affected the measurement.

Better:

Parallax occurred because the scale was viewed at an angle rather than perpendicular to the pointer.

Poor:

Human error affected timing.

Better:

Variation in the observer’s reaction time contributed to the scatter in the manually measured intervals.

Error analysis should describe physical or procedural mechanisms, not vague labels.

## 15.6 Make the analysis quantitative when possible

Weak:

Diameter uncertainty was important.

Better:

The diameter term is approximately 0.50%, compared with only 0.089% from the mass term. The diameter is therefore the more important quantity to improve.

# 16. Correct reporting of numerical results

The standard reporting form is

x=xbest±ux unit.

Examples:

### Poor

ρ=7.15560624 g cm−3.

### Better

ρ=7.16±0.05 g cm−3.

### Poor

L=12.43827±0.2 cm.

### Better

L=12.4±0.2 cm.

### Poor

V=4.2±0.03756 V.

### Better

V=4.20±0.04 V.

# 17. Correct reporting of tables and graphs

A data table should make clear which quantities were measured, which were calculated, what uncertainty applies, and what units are used.

### Poor table

| mass | diameter | density |
| --- | --- | --- |
| 56.2 | 20 | 7.1556 |

### Better table

| Quantity | Value | Uncertainty | Unit |
| --- | --- | --- | --- |
| Mass, m | 56.20 | 0.05 | g |
| Diameter, D | 20.00 | 0.05 | mm |
| Height, h | 25.00 | 0.10 | mm |
| Density, ρ | 7.16 | 0.05 | g cm−3 |

When reporting a fitted graph, interpret the parameter physically.

Poor:

The slope is 48.3.

Better:

Linear regression of V against I gave a slope of 48.3 V A−1, corresponding to a resistance of 48.3 Ω.

# 18. Complete reporting example: simple pendulum

## 18.1 Objective

Investigate the dependence of pendulum period on length and determine the gravitational acceleration g from the slope of a graph of T2 against L.

## 18.2 Physical model

T=2πLg,

so

T2=4π2gL.

## 18.3 Raw and processed data

Preserve the directly measured values of L and t10, then calculate

T=t1010

and T2, propagating the uncertainty at each stage.

## 18.4 Fitting

Fit

T2=mL+c.

From the fitted slope,

g=4π2m.

If the fit gives

m=4.033±0.006 s2 m−1,

then the fitted-slope contribution gives approximately

g=9.79±0.02 m s−2.

## 18.5 Example discussion paragraph

The measured gravitational acceleration is consistent with the appropriate reference value at the precision achieved in the experiment. Manual timing contributes random variation because the observer must react to the passage of the pendulum. Timing multiple oscillations reduces the relative influence of this effect. A systematic contribution may arise if L is measured to the bottom of the bob rather than to its centre of mass. The simple-pendulum relation also assumes small-amplitude motion. The experiment could therefore be improved through automated timing, careful determination of the effective pendulum length, and use of a small reproducible release angle.

## 18.6 Example conclusion

The experiment confirms the expected approximately linear relationship between T2 and L for a simple pendulum. Gravitational acceleration is obtained from the fitted slope. The final value should be reported together with its uncertainty, and agreement with a reference value should be assessed relative to that uncertainty rather than by numerical closeness alone.

# 19. Recommended laboratory report structure

## 19.1 Title

State clearly what was investigated.

## 19.2 Objective

State the physical aim directly.

## 19.3 Apparatus

Identify important equipment and relevant instrument resolution.

## 19.4 Physical principle

Give only the theory required to understand the experiment and data analysis. Define every scientific symbol when it first appears.

## 19.5 Experimental method

Describe what was actually done, especially details that affect measurement quality.

## 19.6 Raw data

Present original observations clearly and preserve units.

## 19.7 Data treatment

Show representative calculations, averages, transformations, uncertainty estimates, and propagation.

## 19.8 Graphical analysis

Show the graph, fitted model, slope, intercept, parameter uncertainties, and physical interpretation.

## 19.9 Final result

Make the principal result easy to locate, for example

x=xbest±ux unit.

## 19.10 Discussion

Discuss:

- whether the result is consistent with the model or reference value;

- the magnitude of the uncertainty;

- dominant uncertainty sources;

- random effects;

- systematic effects;

- model assumptions;

- specific improvements.

## 19.11 Conclusion

Answer the original physical question concisely. Do not introduce new analysis.

# 20. Common beginner mistakes

## 20.1 Reporting only the expected value

The experiment must report what was measured, not simply copy the accepted value.

## 20.2 Excessive significant figures

Do not report calculator digits that are not justified by the uncertainty.

## 20.3 Missing units

A physical result without its required unit is incomplete.

## 20.4 Calling every discrepancy “human error”

Identify the actual physical or procedural mechanism.

## 20.5 Removing inconvenient data

Do not exclude a point without a documented scientific reason.

## 20.6 Equating display resolution with measurement uncertainty

Practical limitations may be much larger than the smallest displayed digit.

## 20.7 Rounding too early

Carry sufficient digits through intermediate calculations and round only at the end.

## 20.8 Transforming data but not uncertainty

If Q=x2, both x and its uncertainty must be propagated to Q.

## 20.9 Forcing a fit through the origin

Do so only when justified by both the model and the experimental circumstances.

## 20.10 Treating percentage difference as complete error analysis

A complete analysis also examines uncertainty, random scatter, systematic effects, and model assumptions.

# 21. Quick-reference formula sheet

## Repeated measurements

x=1Ni=1Nxi

s=i=1Nxi−x2N−1

ux=sN

## Relative and percentage uncertainty

relative uncertainty=uxx

% uncertainty=100uxx

## Addition and subtraction

uQ=uA2+uB2

for Q=A+B or Q=A−B.

## Product of powers

For relatively small, independent uncertainties,

uQQ2≈m2uAA2+n2uBB2+p2uCC2

for Q=AmBnCp.

## General first-order propagation formula

For Q=fx1,x2,…,xn,

uQ2≈i=1n∂Q∂xiuxi2

when the contributing uncertainties are treated as independent. If inputs are correlated, covariance terms are required.

## Percentage difference from a reference

100x−xrefxref%

## Straight-line model

y=mx+c

Always determine the physical meaning and unit of m, and examine whether c is physically reasonable.

# 22. Final analysis and reporting checklist

Before analysing data, check:

- Are all raw observations preserved?

- Are units recorded?

- Is instrument resolution known?

- Which observations are true repeats under the same conditions?

During data treatment, check:

- Why is each transformation being performed?

- Is the transformation physically justified?

- Has the original data been preserved?

- Has the uncertainty been propagated?

- Has premature rounding been avoided?

When analysing uncertainty, check:

- Which quantities contribute uncertainty?

- Are absolute or relative uncertainties required?

- Which contribution dominates?

- Are the uncertainty sources reasonably independent?

When using graphs, check:

- Are axes labelled with quantities and units?

- Are error bars required?

- Is the chosen model physically justified?

- What do the slope and intercept mean?

- Are residuals consistent with the model?

Before reporting, check:

- Is the final uncertainty stated?

- Are significant figures justified?

- Are units correct?

- Is the result compared with theory or a reference using the uncertainty?

- Are error sources specific rather than vague?

- Are proposed improvements connected to identified limitations?

# 23. Essential experimental mindset

Do not ask only:

Did I get the expected answer?

Instead ask:

- What did I actually measure?

- How well did I measure it?

- How were the raw observations transformed?

- How uncertain is the final result?

- Which measurement limits the experiment?

- Does the physical model describe the observations?

- Is the discrepancy significant compared with the uncertainty?

- What specific change would most improve the experiment?

The guiding principle of this manual is

Measure carefully, record honestly, analyse systematically, quantify uncertainty, interpret physically, and report only what the evidence justifies.

# Recommended introductory reference

M. Babayiğit Cinali and M. Cankurtaran (eds.), Introduction to Physics Laboratory and Basic Physics Experiment, Hacettepe University.